GenICam browser usage

| Abstract | This documents explains how to use GenICam browser |
| --- | --- |
| Version | 0.1 |
| Status | Draft |
| Date | 2020/06/25 |

Table of contents

1	Introduction	2

2	Check Version	2

2.1	Launch GenICam Browser	2

2.2	Help > About	3

2.3	Tools > Start Firmware Manager	4

2.4	Firmware revision	5

3	Check Camera	6

3.1	Reset the camera	6

3.2	Configure the camera	9

# Introduction

Check FrameGrabber firmware version is correctly setup
Check the camera is operational

# Check Version

## Launch GenICam Browser

## Help > About

Check version is 11.0.3

## Tools > Start Firmware Manager

Check installed firmware is CoaxlinkQuad G3 (2 camera)

## Firmware revision

Press >

Check the revision of the firmware

# Check Camera

## Reset the camera

In Interfaces section, execute command CxpPoCxpTurnOff.
The Taprisiot should turn off.

Press Update Dev List button: CMV50000_CXP disappears from Remote Devices section

In Interfaces section, execute command CxpPoCxpAuto to restart the camera.
It takes few seconds for the camera to start-up.
Press Update Dev List button; the camera is ready when it appears in Remote Devices.

## Configure the camera

Set DeviceStreamChannelPacketSize to 4096

Set AcquisitionFrameRate to 10

Set ExposureTime (in us).

Press Grab or Start Stream button.

In case of stream, frame counter will increase.